## Article

# AI and Machine Learning Methods for n-Colorant Printer Characterization

Muhammad Hamza Zafar 1,∗ and Phil Green 1

Colourlab, Department of Computer Science, Norwegian University of Science and Technology (NTNU), 2815 Gjøvik, Norway

*	Correspondence: muhammad.h.zafar@ntnu.no

# Abstract	1

- Printer color characterization maps colorant amounts to colorimetry; the	2

classical toolchain strains beyond four inks. We compare 14 machine learning methods	3

against polynomial regression on nine datasets spanning three- to seven-colorant datasets,	4

under cross-validation with CIEDE2000 (∆E00).

Gaussian process regression	11

remains the best-median model on all nine datasets, but at seven inks its margin over	12

a degree-4 CIELAB polynomial is a factor of 1.7. An exploratory probe finds	13

the best unaided large-language-model equation 13 times worse than a least-squares	14

cubic. Machine learning characterizes n > 4 colorant systems well, and so does classical	15

polynomial regression, given the fitting space and degree it needs.	16

Keywords: printer characterization; color; machine learning; CIEDE2000; n-color; printing	17

industry	18

Received:

Revised:

Accepted:

Published:

Copyright: © 2026 by the authors. Submitted to Technologies for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license.

- Introduction	19

The forward model in printer color characterization is the problem of predicting the

colorimetry (normally CIE XYZ or CIE Lab) that a printer produces for a given combination of colorant	21

inputs.Cyan, magenta, yellow, and black (CMYK) process colours are the most common, but larger	22

colorant sets (e.g., adding orange, green, violet, or blue) are also used in  commercial and packaging and	23

printing. Two approaches are widely used, often in combination: a dense	24

look-up table (LUT) is built over the colorant space, interpolated at print time, or a	25

polynomial regression is fitted. . Both approaches require training data from the printing and measurement of test charts; however, they scale poorly	26

as the number of colorants n grows:.A LUT’s node count grows exponentially with n	27

[1], and a fixed-order polynomial’s term count also grows combinatorially (e.g. 20 terms	28

for a 3rd-order expansion of 3 inputs, 35 for 4 inputs), while offering no principled way	29

to add higher-order interaction terms only where the data need them. This has made	30

n > 4 colorant printing (CMYKOGV, CMYKOGB, and similar seven-channel systems used	31

commercially to extend gamut and reduce spot-color dependence [2,3]) a case where the	32

standard characterization toolchain is structurally strained, independent of how much	33

calibration data is available.	34

Machine learning methods that fit a function directly from data, rather than a fixed- 35 degree polynomial basis or a discretized LUT, are an obvious candidate to close this gap, and 36 have been proposed for color device characterization since at least the early 1990s [4,5], and  37 a continuing literature applies neural and ensemble models to CMYK-to-CIELAB printer 38 characterization [6,7]. That literature is fragmented across devices and protocols, however, 39 and rarely evaluates a broad model set under a single corrected, perceptually uniform  40 error metric across colorant counts. This paper is organized around two questions: (a) can AI and machine learning match or beat classical polynomial regression at  43 n ≤ 4 colorants, potentially avoiding gray-component-replacement-style handling of the  44 K channel; and (b) can they handle n > 4 colorant systems, where classical polynomial and  45 LUT-based methods are not designed to scale gracefully. Question (a) is answered with 46 corrected, cross-validated results on three printer datasets at n = 3 and n = 4 (Section 4);  47 question (b) in Section 4.7, with full model comparisons on a five-ink KCMYG set and two  48 independent seven-ink sets (CMYKOGV, CMYKOGB).	49

64

- Related Work	65

Device characterization, mapping a device’s control values to a device-independent	66

color space, is a long-standing problem in color science, surveyed for example in Bala [9] and Green [xx]. The classical	67

solutions are LUT-based interpolation and low-order polynomial regression fit by ordinary	68

least squares, often used in combination..	69

Neural networks were proposed for printer and scanner characterization as early as Kang	70

and Anderson [4] and Tominaga [5], and later work compared them against polynomial	71

transforms for camera characterization [10], finding approximately equal accuracy and	72

recommending the polynomial on practical grounds: it is simpler to fit, and its parameter	73

set is smaller and easier to audit. The line has continued since: wavelet neural networks [6]	74

and stacking ensembles [7] map CMYK to CIELAB directly, and Gaussian process regression	75

has been applied to camera characterization [11]. This paper revisits that comparison with	76

a much larger model set (14 methods spanning linear, kernel, tree-ensemble and neural	77

architectures), a perceptually uniform error metric (Section 3.3), a direct	78

test of n > 4 colorant generalization against a properly fitted classical baseline, and an	79

exploratory test of a large language model as a zero-shot color predictor.	80

Printer characterization and calibration models conventionally	82

take CIELAB as their output space, but evaluating error using a more perceptually uniform metric such as CIEDE2000	83

is preferred He et al. [14] compare XYZ,  90 log-XYZ and CIELAB regression targets for third-order polynomial and neural models  91 on a CMYK 3D printer and found raw XYZ the worst of the three.	98

Characterizing an n > 4 colorant process is not itself a new problem. The analytical  99

line runs from the seven-ink color-to-colorant transforms of Boll [2], through Neugebauer-	100

family models generalized to n inks by barycentric subdivision [15], to the spot color	101

overprint model of Deshpande et al. [16]

Shi et al. [17] train a neural spectral predictor for ten-ink	104

multilayer printing, and Chen and Urban [18] learn deep forward and backward models	105

for a six-material 3D printer.

110

Large language models have been shown to act as capable in-context regressors on	111

numerical data [19] and, when allowed to write and execute code, to discover scientific	112

equations by proposing skeletons whose parameters are fitted numerically [20]. The ex-	113

ploratory probe of Section 4.8 asks the domain-specific form of that question, whether	114

a language model can write a usable CMY→XYZ characterization equation from mea-	115

surement pairs in its prompt, for which the authors have found no published attempt; its	116

code-execution condition reproduces exactly the division of labor that Shojaee et al. [20]	117

formalize.	118

The regression methods compared in Section 3.2 draw on standard machine learning	119

literature: ridge regression [21], the lasso and elastic net [22,23], principal component	120

regression [24], partial least squares [25], support vector regression [26], k-nearest neighbors	121

[27], classification and regression trees [28], random forests [29], gradient boosting [30],	122

and Gaussian process regression [31]. Color differences throughout use the CIEDE2000	123

formula [32], the current standard perceptual color-difference metric.

127

- Materials and Methods	128

  - Datasets	129

This paper is structured around the full n = 3 → 4 → 5 → 7 colorant ladder	130

summarized in Table 1. Three printer characterization datasets (PC10, PC11, FOGRA51)	131

consist of 1,617 measured CMYK combinations under a shared measurement	132

protocol (spectrophotometric CIE Lab and CIE XYZ, D50 illuminant, 2° standard observer), 133 and each were evaluated twice: once restricted to the K = 0 subset with only C, M, Y as  134 inputs (818 measured rows, the n = 3/CMY condition), and once using the full sample  135 set with C, M, Y, K as four inputs (1,617 measured rows, the n = 4/CMYK condition). 136 Two row counts were kept apart throughout, and both are given in Table 1: the counts  137 as received, just quoted, and the counts as analyzed after duplicate rows in CMYK space were 138 removed (Section 3.3), giving 795 for every CMY condition and 1,588 for every 139 CMYK condition, the latter comprising 795 rows with K = 0 plus 793 with K > 0. Every  140

∆E00 statistic in this paper is computed over the analyzed rows; the as-received counts 141 appear only where the datasets themselves are being described. A fourth, n = 4 dataset serves the cross- 145 printer generalization experiments of Section 4.6 rather than the model-comparison tables: 146 13 newsprint press runs, 1,485 CMYK samples per run, 147 measured as 36-band spectral reflectance (380–730 nm) and converted to CIE XYZ (D50/2°) 148 via the colour-science library [35]. Registered characterization data sets such as FOGRA52 and APTECH are usually derived from multiple press runs and subject to smoothing, as opposed to the newsprint datasets which are individual measured press runs and subject to greater noise from print and measurement variability. It has previously been noted (Green 2006) that averaging and smoothing lead to smaller model errors.	151

Three further datasets extend the ladder to n = 5 and n = 7 (Section 4.7). KCMYG  152

is a five-ink (black, cyan, magenta, yellow, green) chart of 2,214 patches, measured as	153

36-band spectral reflectance and converted to XYZ. CMYKOGV is the	154

APTEC seven-ink (CMYK plus orange, green, violet) coated chart published through the	155

ICC characterization-data registry [36], measurements in XYZ and Lab (D50/2°, M1)	156

with 3,534 patches; 232 of these are duplicates

which were removed by the	158

same exact-duplicate policy applied to every dataset compared in this paper (Section 3.3),	159

leaving 3,302 analyzed samples. CMYKOGB is a seven-ink (CMYK plus orange,	160

green, blue) chart of 2,000 spectral patches. The two	161

seven-ink sets differ in their seventh colorant (violet vs. blue) and come from different	162

providers.

164

Table 1. Datasets across the full n = 3 → 4 → 5 → 7 colorant ladder. Rows are given as received, with the analyzed count in parentheses where byte-identical duplicate rows are removed (Section 3.3);. Analyzed rows are pooled across all 5 cross-validation folds, so every one of them is predicted exactly once.

| Dataset | Substrate | Colorants | n | Rows | Source |
| --- | --- | --- | --- | --- | --- |
| PC10 | CCNB 250 g/m2 (PS 11) | CMY | 3 | 818 (795) | APTEC PC10 [34] |
| PC10 | CCNB 250 g/m2 (PS 11) | CMYK | 4 | 1,617 (1,588) | APTEC PC10 [34] |
| PC11 | CCNB 250 g/m2 (PS 11) | CMY | 3 | 818 (795) | APTEC PC11 [34] |
| PC11 | CCNB 250 g/m2 (PS 11) | CMYK | 4 | 1,617 (1,588) | APTEC PC11 [34] |
| FOGRA51 | Reference (ISO 12647-2) | CMY | 3 | 818 (795) | Fogra Research Institute [33] |
| FOGRA51 | Reference (ISO 12647-2) | CMYK | 4 | 1,617 (1,588) | Fogra Research Institute [33] |
| News | Newsprint, 13 press runs | CMYK | 4 | 1,485 / run | Newspaper industry |
| KCMYG | Coated (5-ink) | CMYK+G | 5 | 2,214 | Commercial printer |
| CMYKOGV | Coated (7-ink) | CMYK+OGV | 7 | 3,534 (3,302) | APTEC, ICC registry [36] |
| CMYKOGB | Coated (7-ink) | CMYK+OGB | 7 | 2,000 | Commercial printer |

  - Models	165

Fourteen regression methods are compared (Table 2)

All models are implemented input-dimension-agnostically, so the	170

same code path serves the n = 3, n = 4, n = 5, and n = 7 conditions. Every stochastic	171

model is seeded (fixed seed 42) for reproducibility.	172

Four further variants are added for Section 4.5 and appear in the same tables,	173

listed at the foot of Table 2.  Two of them lift the third-order cap (poly4) and the	174

XYZ fitting space (poly3_cbrt) separately, one lifts both (poly4_cbrt), and the fourth	175

(gaussian_process_cbrt) offers the same change of fitting space to the polynomial’s	176

strongest competitor.

The _cbrt variants differ from their plain counterparts in one respect:	180

the estimator is fitted against √3 XYZ (physical XYZ, clipped at zero) and its prediction is	181

cubed on the way out.	182

Table 2. Methods implemented.

# Model	Configuration

Polynomial regression (degree 3)	Full 3rd-order feature expansion + ordinary least squares Ridge regression [21]	 α = 0.5

Lasso regression* [22]	α = 10−3

Elastic Net* [23]	α = 10−3, ℓ1 ratio = 0.5

Principal component regression [24]	All components retained + Ridge (α = 0.5)

Partial least squares regression [25]	3 components

k-nearest neighbors [27]	k = 5, uniform weights

Support vector regression* [26]	RBF kernel, C = 10, γ = scale, ε = 0.01 Decision tree [28]	Unconstrained depth

Random forest [29]	200 trees, max depth 15

Gradient boosting [30]	200 estimators, learning rate 0.05, max depth 5

×

Gaussian process regression [31]	Const	RBF + WhiteKernel(10−3, bounds 10−9–105), y-normalized, 15 opti-

mizer restarts

MLP (shallow)	1 hidden layer, 64 units, L-BFGS

MLP (deep)	3 hidden layers, 64 units each, L-BFGS

√

Fitting-space and degree variants (Section 4.5)

poly3_cbrt	Degree-3 expansion + OLS, fitted against 3

XYZ; prediction cubed back

√

poly4	Full 4th-order feature expansion + ordinary least squares, fitted on XYZ

√

poly4_cbrt	Degree-4 expansion + OLS, fitted against 3 XYZ; prediction cubed back Gaussian process as configured above, fitted against 3 XYZ; prediction cubed

gaussian_process_cbrt

back

## Evaluation Protocol	183

Each (dataset, model) pair is evaluated with 5-fold cross-validation (fixed seed 42,	184

shuffled). Duplicate rows

were removed.	187

191

The newsprint runs are deliberately not de-duplicated: 205 their repeated colorant entries carry differing measurements (roughly 0.4–1.9 ∆E00 of 206 press repeatability, Section 4.6), which is signal about the press rather than an artifact, so 207 those repeats are retained and made to co-travel into one fold by grouped cross-validation 208 instead. Within each fold:	209

- A MinMax scaler is fit on the training fold’s inputs and, separately, on the training  210

fold’s XYZ targets; scaling parameters are never estimated from the held-out fold.   211

- The model is fit in the normalized (0–1) input/output space.	212

- Predictions are generated in normalized space, then inverse-transformed back to  213

XYZ using the training fold’s target scaler.	214

- Predicted XYZ values are clipped at zero (tristimulus values cannot be negative).    215

- Both predicted and ground-truth XYZ are converted to CIE Lab under the D50 illumi-  216

nant and 2° standard observer, and the per-sample color difference is computed with  217

the CIE 2000 formula [32].	218

Every sample in a dataset is held out exactly once across the 5 folds, so the reported	219

statistics pool over the entire dataset rather than a single train/test split. Summary statistics	220

are the median, 95th percentile, and maximum of the per-sample ∆E00 distribution, not	221

the mean or standard deviation, since color-difference errors are not normally distributed	222

and a small number of large outliers can otherwise dominate a mean-based summary.	223

Two further protocol details are specific to the n = 5 and n = 7 datasets. First,	224

their folds are assigned with scikit-learn’s GroupKFold, keyed on the distinct input recipe	225

(rounded to 6 decimal places). KCMYG and CMYKOGB contain repeated recipes whose	226

measurements genuinely differ (CMYKOGB’s 20 repeated paper-white patches spread	227

by 0.348 ∆E00), so, as with the newsprint data, those repeats are kept as measured and forced into a	228

single fold rather than removed; CMYKOGV-7’s duplicates are byte-identical and are	229

removed by the policy above

. GroupKFold’s assignment is deterministic and unshuffled, in contrast to the	231

seeded shuffled k-fold used for the coated n ≤ 4 sets. Second, Gaussian process training	232

folds are subsampled to at most 2,000 rows (fixed seed 42) to bound the method’s O(N3) fit	233

cost, a cap that only CMYKOGV-7 (3,302 analyzed rows) actually reaches.	234

239

Reproducibility.	240

All models use fixed seeds and a pinned dependency set (Python 3.12.11, scikit-learn	241

1.7.2, NumPy 2.3.3, SciPy 1.16.2)

.

The Gaussian process	244

uses the single dataset-agnostic kernel configuration of Table 2 (WhiteKernel initialized	245

at 10−3, bounds 10−9–105, 15 optimizer restarts), with no per-dataset tuning

. Two checks bound how far the numbers travel, and both were re-run on	247

the de-duplicated data used throughout this paper. Re-running the 16 models then in	248

the registry on the six coated datasets (96 model/dataset cells) on a second machine	249

(Linux/x86-64, OpenBLAS) under an identical pinned environment, same code, data and	250

seeds, reproduces the macOS/arm64 (Apple Accelerate) values on 58 of 96 cells exactly	251

and on 76 of 96 to within 0.01 ∆E00, with a worst case of 0.108; each of the largest	252

divergences is an iteratively optimized neural net, while the closed-form, kernel and tree	253

methods all agree exactly or to within 0.01. Pinned dependencies alone therefore do not	254

give cross-platform bit-reproducibility, and the reported values are additionally pinned to	255

one platform (macOS/arm64). The Gaussian process is the least platform-sensitive of the	256

family: its final configuration reproduces exactly on seven of the nine non-newsprint datasets and	257

to within 0.016 on the other two, and all 182 newsprint Gaussian process results (13 within-run,	258

156 cross-run, 13 leave-one-out) are bit-exact across the two platforms. Varying the cross-	259

validation shuffle seed over 0/1/2/3/42 on the six coated datasets moves each model’s	260

median by a standard deviation of at most 0.001 (Gaussian process), 0.009 (polynomial),	261

0.023 (SVM), 0.044 (k-NN) and 0.091 (deep MLP), so fold assignment is not a meaningful	262

source of variance at the precision reported here, the MLPs excepted.

A	265

third and stronger check is independent of this implementation entirely: a clean-room	266

re-implementation, carried out by an independently tasked AI coding agent in a separate	267

workspace, built from the raw measurement files and a prose specification alone (with	268

no access to our code, configuration or results), wrote its own CGATS parser, Lab/XYZ	269

and spectral-to-XYZ conversions, cross-validation and ∆E00 path, and reproduced our	270

Gaussian process medians to within 0.003 ∆E00 and the third-order polynomial’s to within	271

0.001 on every coated dataset.

276

Use of generative AI in conducting this study.	277

Generative AI assisted the implementation of the pipeline, the figure-plotting code	278

and the manuscript prose, as declared in the Acknowledgments. No reported quantity orig-	279

inates with an assistant: every number in this paper is produced by the committed pipeline	280

from the committed data, written to a version-controlled results CSV and checked against	281

it, with the clean-room reimplementation above as a further check on the implementation	282

itself. This use of AI as an instrument is distinct from Section 4.8, where large language	283

models are the object of study; no model evaluated there had any role in producing the rest	284

of the paper.	285

Correction to the AIC 2025 conference results.	286

We note that a previous version of this analysis [8] computed ∆E00 on normalized	287

(0–1 scaled) XYZ values rather than on the physical 0–100 XYZ scale, which understated	288

the reported color differences by approximately a factor of five. All results reported in this	289

paper are corrected and supersede the previous published results.	290

- Results	291

  - n = 3 (CMY)	292

Table 3 reports median, 95th-percentile, and maximum ∆E00 for all 14 registry models	293

plus the four fitting-space variants on the three n = 3 (CMY) datasets, pooled across	294

the 5 cross-validation folds so that every one of the 795 analyzed K = 0 samples per	295

dataset is predicted exactly once. Figure 1 ranks the 14 registry medians on a log scale	296

against ∆E00 = 1. Gaussian process	297

regression is the best-median model on all three datasets by a wide margin, fitted in	298

cube-root space (0.041, 0.043, 0.053) slightly ahead of fitted in XYZ (0.046, 0.046, 0.057).	299

Polynomial regression is a clear second, but results vary according to its	300

degree and its fitting space

306

Table 3. Median, 95th-percentile, and maximum ∆E00 per model for the n = 3 (CMY) condition on the three printer datasets, 795 analyzed K = 0 samples each (818 as received, less 23 byte-identical duplicates; Section 3.3). All values from 5-fold cross-validation with every sample predicted exactly once; ∆E00 computed on denormalized XYZ (D50 illuminant, 2° standard observer), per Section 3.3. Models are sorted by PC10 median; the four fitting-space variants of Section 4.5 are marked †. Source: journal/results/{PC10,PC11,FOGRA51}-CMY/summary.csv.

PC10	PC11	FOGRA51

| Model | Median | P95 | Max |  | Median | P95 | Max |  | Median | P95 | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gaussian Process (cbrt)† | 0.041 | 0.148 | 2.270 |  | 0.043 | 0.131 | 1.749 |  | 0.053 | 0.165 | 0.458 |
| Gaussian Process | 0.046 | 0.153 | 2.274 |  | 0.046 | 0.139 | 1.747 |  | 0.057 | 0.182 | 0.679 |
| Polynomial (4th order, cbrt)† | 0.085 | 0.255 | 2.171 |  | 0.084 | 0.226 | 1.789 |  | 0.156 | 0.341 | 1.168 |
| Polynomial (4th order)† | 0.101 | 0.340 | 2.331 |  | 0.097 | 0.325 | 1.680 |  | 0.125 | 0.410 | 1.846 |
| Polynomial (3rd order, cbrt)† | 0.148 | 0.454 | 2.048 |  | 0.140 | 0.341 | 1.653 |  | 0.432 | 0.916 | 1.958 |
| Polynomial (3rd order) | 0.268 | 0.982 | 6.795 |  | 0.238 | 0.855 | 7.838 |  | 0.369 | 1.135 | 8.251 |
| SVM | 0.730 | 1.835 | 7.489 |  | 0.693 | 1.831 | 5.466 |  | 0.796 | 1.913 | 6.107 |
| Gradient Boosting | 0.901 | 3.011 | 25.767 |  | 0.801 | 2.600 | 11.474 |  | 0.834 | 3.491 | 16.821 |
| MLP (Deep) | 1.134 | 4.072 | 15.654 |  | 1.132 | 3.767 | 22.840 |  | 1.083 | 4.223 | 13.447 |
| MLP (Shallow) | 1.265 | 4.777 | 20.787 |  | 1.137 | 4.020 | 9.511 |  | 1.304 | 4.934 | 20.615 |
| Random Forest | 1.731 | 4.040 | 10.446 |  | 1.710 | 4.181 | 10.425 |  | 1.887 | 4.316 | 10.982 |
| k-NN | 1.998 | 4.210 | 13.466 |  | 1.923 | 4.087 | 15.360 |  | 1.905 | 4.180 | 13.354 |
| Decision Tree | 4.410 | 9.723 | 16.067 |  | 4.327 | 9.040 | 15.703 |  | 4.412 | 9.425 | 18.467 |
| Lasso | 6.478 | 24.696 | 44.001 |  | 6.151 | 23.682 | 45.374 |  | 6.642 | 25.267 | 42.222 |
| PLSR | 6.520 | 24.734 | 43.855 |  | 6.161 | 23.939 | 45.211 |  | 6.650 | 25.305 | 42.061 |
| Elastic Net | 6.532 | 24.777 | 43.927 |  | 6.143 | 23.591 | 45.294 |  | 6.640 | 25.312 | 42.144 |
| Ridge | 6.545 | 24.698 | 43.854 |  | 6.148 | 23.799 | 45.213 |  | 6.658 | 25.175 | 42.065 |
| PCR | 6.545 | 24.698 | 43.854 |  | 6.148 | 23.799 | 45.213 |  | 6.658 | 25.175 | 42.065 |

Figure 1. Median ∆E00 per model across the three CMY datasets (795 analyzed K = 0 samples each, 5-fold cross-validation), ranked by mean median across datasets; the two ∆E00-refined polynomial variants of Section 4.4 are shown alongside the 14 registry models. Log scale;).

  - n = 4 (CMYK) and the n=3 → n=4 Comparison	307

Table 4 reports the same statistics for the n = 4 (CMYK) condition, using the full	308

sample set of 1,588 analyzed rows, the 795 K = 0 rows of Table 3 plus 793 rows with	309

K > 0, with K retained as a fourth input rather than dropped. Gaussian process regression	310

remains the best-median model on all three datasets.

The relative ordering of the remaining models	314

is otherwise stable between n = 3 and n = 4 (SVM third, gradient boosting fourth, on all	315

three datasets)

.	317

Table 4. Median, 95th-percentile, and maximum ∆E00 per model for the n = 4 (CMYK) condi-tion on the three printer datasets, 1,588 analyzed samples each (795 with K = 0 plus 793 with K > 0; 1,617 as received, less 29 byte-identical duplicates). Same protocol as Table 3. Models are sorted by PC10 median; the four fitting-space variants of Section 4.5 are marked †. Source: journal/results/{PC10,PC11,FOGRA51}-CMYK/summary.csv.

PC10	PC11	FOGRA51

| Model | Median | P95 | Max |  | Median | P95 | Max |  | Median | P95 | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gaussian Process (cbrt)† | 0.050 | 0.229 | 2.723 |  | 0.052 | 0.216 | 1.766 |  | 0.070 | 0.213 | 1.412 |
| Gaussian Process | 0.056 | 0.471 | 7.227 |  | 0.057 | 0.383 | 7.684 |  | 0.069 | 0.267 | 3.153 |
| Polynomial (4th order, cbrt)† | 0.112 | 0.387 | 2.625 |  | 0.112 | 0.380 | 2.709 |  | 0.167 | 0.484 | 3.642 |
| Polynomial (3rd order, cbrt)† | 0.219 | 0.759 | 3.506 |  | 0.216 | 0.752 | 3.511 |  | 0.437 | 1.097 | 5.786 |
| Polynomial (4th order)† | 0.246 | 2.811 | 25.385 |  | 0.227 | 2.464 | 23.535 |  | 0.213 | 1.772 | 19.665 |
| Polynomial (3rd order) | 0.942 | 8.310 | 33.978 |  | 0.869 | 7.500 | 33.070 |  | 0.816 | 6.733 | 29.287 |
| SVM | 1.017 | 5.387 | 23.175 |  | 0.994 | 4.731 | 19.743 |  | 0.996 | 4.872 | 13.246 |
| Gradient Boosting | 1.541 | 6.959 | 29.019 |  | 1.410 | 5.721 | 25.210 |  | 1.508 | 6.578 | 30.742 |
| MLP (Deep) | 1.729 | 11.800 | 48.645 |  | 1.633 | 9.516 | 29.132 |  | 1.658 | 9.675 | 37.063 |
| MLP (Shallow) | 2.002 | 10.385 | 29.829 |  | 1.879 | 11.921 | 27.502 |  | 1.957 | 9.850 | 29.364 |
| k-NN | 2.229 | 5.839 | 12.798 |  | 2.132 | 5.585 | 11.064 |  | 2.216 | 5.999 | 22.229 |
| Random Forest | 2.376 | 4.980 | 10.499 |  | 2.349 | 4.791 | 12.587 |  | 2.635 | 5.630 | 10.592 |
| Decision Tree | 4.611 | 9.648 | 24.345 |  | 4.413 | 9.098 | 22.321 |  | 4.493 | 10.048 | 22.887 |
| Ridge | 8.876 | 24.436 | 42.282 |  | 8.443 | 23.337 | 41.640 |  | 8.788 | 24.822 | 40.642 |
| PCR | 8.876 | 24.436 | 42.282 |  | 8.443 | 23.337 | 41.640 |  | 8.788 | 24.822 | 40.642 |
| Elastic Net | 8.876 | 24.373 | 42.449 |  | 8.446 | 23.364 | 41.816 |  | 8.780 | 25.045 | 41.056 |
| Lasso | 8.886 | 24.479 | 42.589 |  | 8.394 | 23.617 | 41.195 |  | 8.699 | 25.264 | 41.372 |
| PLSR | 8.887 | 25.113 | 42.276 |  | 8.476 | 23.647 | 41.592 |  | 8.596 | 25.176 | 42.190 |

Figure 2 compares median ∆E00 at n = 3 (CMY) versus n = 4 (CMYK) for every 318 registry model on PC10, the full 1,588-sample set now including the K > 0 rows. Adding  319 the fourth input channel is not uniformly costly: third-order polynomial regression fitted on 320 XYZ, which has no mechanism to represent the extra channel’s interaction terms beyond its 321 fixed cubic expansion, degrades from a median of 0.268 to 0.942 (×3.5), whereas Gaussian 322 process regression is comparatively robust, degrading only from 0.046 to 0.056 (×1.2).    323

332

Figure 2. Median ∆E00 per registry model on PC10 at n = 3 (CMY, 795 analyzed samples) versus

n = 4 (CMYK, 1,588 analyzed samples). Third-order polynomial regression fitted on XYZ degrades

×3.5 with the added K channel, while Gaussian process regression degrades only ×1.2, the two highlighted comparisons.

## Fold Assignment and the Measurement Noise Floor	333

Gaussian process regression is the best-median model in every dataset/condition	334

combination reported above, and, anticipating Section 4.7, on all nine datasets in this paper:	335

fitted in cube-root space on eight of them and in XYZ on the ninth (FOGRA51-CMYK, by	336

0.001 ∆E00). That makes it the one most worth stress-testing. Two checks bear on it, both	337

read from journal/results/gp_verification.csv, whose k-fold column reproduces the	338

gaussian_process and poly3 rows of Tables 3–4 exactly.	339

Fold assignment. Repeating the six coated conditions with GroupKFold’s deterministic,	340

unshuffled assignment in place of the seeded shuffled split moves GP’s median by at most	341

0.003 ∆E00 (grouped 0.043–0.066 vs. k-fold 0.046–0.069) and third-order polynomial regres-	342

sion’s by at most 0.008 (grouped 0.233–0.938 vs. k-fold 0.238–0.942).

349

Noise floor. The coated sets no longer support a repeatability estimate, because their	350

duplicate pairs were byte-identical: a repeatability of 0.000 measures the export, not the	351

press, and such rows are removed rather than reported. Where repeated patches genuinely	352

differ the estimate is meaningful, and there the Gaussian process sits on it: 0.348 ∆E00	353

across CMYKOGB-7’s 20 paper-white repeats, and 0.38–1.93 per newsprint press run, against	354

per-run GP medians of 0.674–2.141 (Section 4.6).	355

  - Direct ∆E00 Loss	356

The plain polynomial fits reported so far minimize ordinary least squares on normal-	357

ized XYZ, a proxy for the more perceptually-uniform ∆E00 metric the paper actually evaluates on. This	358

experiment tests whether refining the third-order polynomial’s coefficients directly on 359 mean ∆E00 closes any of that gap. Starting from the 363 least-squares solution, the coefficients are refined with a derivative-free optimizer whose 364 objective is mean ∆E00 on the training fold, evaluated in XYZ; there  365 are 60 coefficients at n = 3 (20 third-order terms × 3 output channels) and 105 at n = 4.  366 The refined model is then evaluated with the same seeded 5-fold cross-validation as every 367 other model. Two optimizers are compared: Nelder–Mead (poly3_de00_nm, budget 2,000 368 iterations) and Powell (poly3_de00_powell, budget 200 iterations). Both are reported at  369 n = 3 and n = 4 (Table 5).	370

Table 5. Median, 95th-percentile, and maximum ∆E00 for the least-squares polynomial (poly3) versus its two ∆E00-refined variants, at n = 3 (CMY, 795 analyzed samples) and n = 4 (CMYK, 1,588 analyzed samples) on the three printer datasets. Source: journal/results/{PC10,PC11,FOGRA51}-{CMY,CMYK}/summary.csv.

PC10	PC11	FOGRA51

| Model | Median | P95 | Max |  | Median | P95 | Max |  | Median | P95 | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n = 3 (CMY) |  |  |  |  |  |  |  |  |  |  |  |
| Poly3 (least squares) | 0.268 | 0.982 | 6.795 |  | 0.238 | 0.855 | 7.838 |  | 0.369 | 1.135 | 8.251 |
| Poly3 + ∆E00 (Nelder–Mead) | 0.261 | 1.019 | 5.783 |  | 0.231 | 0.861 | 6.564 |  | 0.360 | 1.156 | 6.198 |
| Poly3 + ∆E00 (Powell) | 0.261 | 0.861 | 5.699 |  | 0.249 | 0.720 | 5.391 |  | 0.372 | 1.041 | 4.018 |
| n = 4 (CMYK) |  |  |  |  |  |  |  |  |  |  |  |
| Poly3 (least squares) | 0.942 | 8.310 | 33.978 |  | 0.869 | 7.500 | 33.070 |  | 0.816 | 6.733 | 29.287 |
| Poly3 + ∆E00 (Nelder–Mead) | 0.894 | 6.896 | 31.639 |  | 0.843 | 6.127 | 31.084 |  | 0.768 | 6.126 | 31.022 |
| Poly3 + ∆E00 (Powell) | 0.867 | 6.075 | 32.666 |  | 0.789 | 5.987 | 28.569 |  | 0.785 | 5.471 | 31.234 |

At n = 3 the effect is confined to the tail. Powell reduces max error by  371 16–51% relative to least squares (PC10 −16%, PC11 −31%, FOGRA51 −51%) and improves 372 P95 on all three datasets, while leaving the median unchanged to within 0.011 ∆E00 in either  373 direction. Nelder–Mead also cuts max error on all three (15–25%) but is marginally worse 374 than least squares at P95 on all three, on a training budget an order of magnitude larger 375 (2,000 vs. 200 iterations); Powell is therefore the preferred variant. At n = 4, where the  376 least-squares polynomial starts out roughly 3.5× worse (Section 4.2), the same refinement is 377 no longer tail-only: Powell lowers the median by 4–9% (0.942 → 0.867, 0.869 → 0.789, 0.816  378

→ 0.785) and P95 by 19–27%, while its effect on the single worst sample turns inconsistent 379 (−4% on PC10, −14% on PC11, +7% on FOGRA51); Nelder–Mead improves the median  380 by 3–6% and P95 by 9–18% there. The refinement buys accuracy in the upper tail while the 381 cubic basis still fits the data comfortably at n = 3, and across the bulk of the distribution  382 once that basis is under real strain at n = 4 (Figure 3).	383

The natural reading of that pattern (that least squares on XYZ is misweighted relative	384

to the perceptual metric, and that optimizing the metric directly repairs the mismatch) is	385

tempting, and Section 4.5 shows it to be wrong. Every improvement recorded in Table 5	386

is smaller than the improvement obtained by leaving the objective alone and changing	387

only the space the least-squares fit is carried out in (compare Powell’s PC10-CMYK median	388

of 0.867 with the same cubic’s 0.219 fitted in cube-root space), and the explicit control	389

for metric alignment in Section 4.5 fails. Direct ∆E00 refinement is best read as a small,	390

expensive correction that a well-chosen fitting space largely subsumes.	391

Figure 3. Maximum (worst-case) ∆E00 for the least-squares polynomial versus its Powell-refined variant across the three n = 3 (CMY) datasets, with each pairing’s medians annotated below the bars. The full median/P95/max statistics for both optimizers, at n = 3 and n = 4, are in Table 5.

## The Fitting Space: A Correction to the Classical Baseline	392

Every polynomial result reported above

was obtained by minimizing squared error on XYZ. That fitting space is a model-	394

ing choice rather than a neutral default; this section measures its cost, establishes the	395

mechanism, and states what the correction changes about the paper’s argument.	396

What was changed, and why it is the same as fitting in CIELAB.	397

The variant poly3_cbrt differs from poly3 in one respect:

the regression	399

target is √3 XYZ instead of XYZ, and the prediction is cubed	400

before scoring. This is not an arbitrary transform. CIELAB is an invertible linear map of	401

the triple ( f (X/Xn), f (Y/Yn), f (Z/Zn)), where f is a cube root above a small threshold;	402

ordinary least squares with a shared design matrix is equivariant under an invertible linear	403

map of the target. (In fact the linear segment has the effect of making the power function approximately 2.4 rather than 3.0 over the domain). Thus fitting to √3 XYZ and fitting in CIELAB are very similar estimators,

Fitting directly in	406

CIELAB confirms this numerically: median ∆E00 0.148 by both routes on PC10-CMY, and	407

0.830 (cube root) against 0.832 (CIELAB) on CMYKOGV-7

.	409

The size of the effect.	410

Table 6 gives the medians and maxima for both polynomial degrees and for the	411

Gaussian process, each fitted both ways, on all nine datasets.  Refitting the degree-3	412

polynomial in cube-root space improves its maximum error on 9 of 9 datasets, by 38–90%,	413

and its median on 8 of 9; the exception is FOGRA51-CMY, where the median rises from	414

0.369 to 0.432 while the maximum still falls from 8.251 to 1.958. The effect grows with	415

ink count, from a factor of 1.8 in the median at n = 3 on PC10 to a factor of 6.5 at n = 7	416

on CMYKOGV. The degree-4 polynomial and the Gaussian process each improve their	417

median on 8 of 9 datasets under the same change, so this is a property of the data rather	418

than of the polynomial, and the correction must be offered to every model before any two	419

are compared.	420

Table 6. Fitting space, all nine datasets: median and maximum ∆E00 for the degree-3 polynomial, the degree-4 polynomial and the Gaussian process, each fitted on XYZ and on √3 XYZ (equivalently, in

CIELAB). Folds, seeds, term counts and the ∆E00 path are identical within each pair. Best median per dataset in bold. Source: journal/results/*/summary.csv.

Polynomial, degree 3	Polynomial, degree 4	Gaussian process

| Dataset | XYZ | cbrt |  | XYZ | cbrt |  | XYZ | cbrt |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Median ∆E00 |  |  |  |  |  |  |  |  |  |
| PC10-CMY | 0.268 | 0.148 | 0.101 |  | 0.085 | 0.046 |  | 0.041 |  |
| PC11-CMY | 0.238 | 0.140 | 0.097 |  | 0.084 | 0.046 |  | 0.043 |  |
| FOGRA51-CMY | 0.369 | 0.432 | 0.125 |  | 0.156 | 0.057 |  | 0.053 |  |
| PC10-CMYK | 0.942 | 0.219 | 0.246 |  | 0.112 | 0.056 |  | 0.050 |  |
| PC11-CMYK | 0.869 | 0.216 | 0.227 |  | 0.112 | 0.057 |  | 0.052 |  |
| FOGRA51-CMYK | 0.816 | 0.437 | 0.213 |  | 0.167 | 0.069 |  | 0.070 |  |
| KCMYG-5 | 1.457 | 1.024 | 1.057 |  | 0.853 | 0.867 |  | 0.764 |  |
| CMYKOGV-7 | 5.386 | 0.830 | 2.080 |  | 0.272 | 0.249 |  | 0.160 |  |
| CMYKOGB-7 | 3.332 | 1.611 | 1.698 |  | 1.132 | 1.280 |  | 0.977 |  |
| Maximum ∆E00 |  |  |  |  |  |  |  |  |  |
| PC10-CMY | 6.795 | 2.048 | 2.331 |  | 2.171 | 2.274 |  | 2.270 |  |
| PC11-CMY | 7.838 | 1.653 | 1.680 |  | 1.789 | 1.747 |  | 1.749 |  |
| FOGRA51-CMY | 8.251 | 1.958 | 1.846 |  | 1.168 | 0.679 |  | 0.458 |  |
| PC10-CMYK | 33.978 | 3.506 | 25.385 |  | 2.625 | 7.227 |  | 2.723 |  |
| PC11-CMYK | 33.070 | 3.511 | 23.535 |  | 2.709 | 7.684 |  | 1.766 |  |
| FOGRA51-CMYK | 29.287 | 5.786 | 19.665 |  | 3.642 | 3.153 |  | 1.412 |  |
| KCMYG-5 | 65.742 | 29.230 | 55.860 |  | 39.488 | 31.738 |  | 16.172 |  |
| CMYKOGV-7 | 59.833 | 27.979 | 57.988 |  | 18.612 | 49.757 |  | 5.713 |  |
| CMYKOGB-7 | 48.974 | 30.274 | 48.974 |  | 24.096 | 44.305 |  | 14.270 |  |

The mechanism is approximability, not the perceptual metric.	421

The tempting explanation is that a cube root brings least squares into line with	422

CIEDE2000, since ∆E00 is computed in a space whose lightness axis is essentially a cube	423

root. That explanation is testable and it fails. To first order, fitting √3 y by unweighted	424

least squares is equivalent to fitting y with sample weights y−4/3; running exactly that	425

weighted fit in XYZ, at the same degree and folds and with each channel weighted	426

by its own yc−4/3, makes the median worse than the unweighted XYZ fit on 8 of the 9	427

datasets (PC10-CMYK 0.942 → 1.551, PC11-CMYK 0.869 → 1.400, FOGRA51-CMYK 0.816	428

→ 1.315), and on all nine it is far worse than the cube-root fit it was meant to imitate	429

(journal/results/weighting_control.csv). Metric alignment is therefore not what is	430

happening.	431

What is happening is that the ink-to-color response is much closer to a low-order	432

polynomial after a compressive transform than before it. Measured as degree-3 residual	433

RMS expressed as a percentage of the target’s standard deviation, on PC10-CMY the per-	434

channel figures fall from 0.99/1.05/1.58 (X/Y/Z) in XYZ to 0.65/0.62/0.75 under a cube	435

root, and on CMYKOGV-7 from 11.88/13.29/15.87 to 3.15/3.02/3.59 (journal/results/	436

residual_rms.csv). The transform is doing approximation-theoretic work, not perceptual	437

work. This also explains the single median regression: FOGRA51-CMY is the one dataset	438

where the cube root makes the polynomial fit worse on every channel (0.95/1.05/1.56 →	439

1.30/1.28/1.65), so there the transform reallocates error from the dark patches, where it	440

wins, to the light ones.	441

The cube root is not privileged.	442

If the mechanism is variance stabilization rather than perceptual alignment, then	443

other compressive transforms should work too, and they do. Table 7 sweeps five target	444

transforms at degree 3. A square root gives a better median than the cube root on 4 of	445

the 9 datasets, and taken across the whole sweep the cube root is the outright winner on  446

only 2, against 3 each for √y and y0.25. Each of the four non-identity transforms beats the  447

identity on 8 of 9 datasets, all four failing on the same one, FOGRA51-CMY. The practical 448 recommendation is therefore not “use a cube root” but “do not fit in XYZ”; the cube root  449 has the incidental advantage of being the one whose fit is exactly the CIELAB fit, which is 450 what makes it reportable as an equivalence rather than as a tuned hyperparameter. This 451 output-side transform should not be confused with root-polynomial regression [37], which 452 takes fractional powers of the input monomials to obtain exposure invariance in camera 453 color correction; here the monomial basis is untouched and the transform is applied to the 454 regression target.	455

Table 7. Median ∆E00 of the degree-3 polynomial fitted against five different target transforms, identical folds and term counts throughout. Best per row in bold. The identity and cube-root columns reproduce the poly3 and poly3_cbrt rows of Table 6 exactly, which is the sweep’s internal control. Source:  journal/results/transform_sweep.csv.

| Dataset identity √y |  |  | √3 y | y0.25 log(1 + y) |  |
| --- | --- | --- | --- | --- | --- |
| PC10-CMY | 0.268 | 0.127 | 0.148 | 0.166 | 0.234 |
| PC11-CMY | 0.238 | 0.124 | 0.140 | 0.154 | 0.206 |
| FOGRA51-CMY | 0.369 | 0.376 | 0.432 | 0.467 | 0.562 |
| PC10-CMYK | 0.942 | 0.252 | 0.219 | 0.250 | 0.385 |
| PC11-CMYK | 0.869 | 0.243 | 0.216 | 0.251 | 0.379 |
| FOGRA51-CMYK | 0.816 | 0.386 | 0.437 | 0.490 | 0.679 |
| KCMYG-5 | 1.457 | 1.045 | 1.024 | 1.023 | 1.112 |
| CMYKOGV-7 | 5.386 | 1.253 | 0.830 | 0.811 | 1.126 |
| CMYKOGB-7 | 3.332 | 1.821 | 1.611 | 1.560 | 1.592 |

Degree is the second lever, and it does not overfit.	456

The polynomial baseline in this comparison was also capped at third order, following	457

standard practice for device characterization: Bala [9] works its regression example at third	458

order and recommends the smallest term count that fits the curvature, and the systematic	459

polynomial study of Hong et al. [38] likewise recommends third-order models. At seven	460

inks that cap is doing a great deal of the work. Table 8 sweeps degrees 2 to 5 on CMYKOGV-	461

7 in both spaces. A degree-4 polynomial fitted in CIELAB reaches a median of 0.272 ∆E00,	462

against 5.386 for the degree-3 XYZ fit that the uncorrected comparison used: a factor of 20	463

attributable entirely to two modeling choices and none of it to the data or to the method’s	464

expressive limits.	465

The obvious objection is overfitting: 330 terms per channel is 990 coefficients against	466

3,302 samples, or 2.7 training rows per fitted coefficient under 5-fold cross-validation. The	467

evidence is against it. The gap between the in-fold training median and the held-out	468

median, averaged over the five folds, is +0.029 ∆E00 at degree 4, unchanged from +0.029	469

at degree 3 and still only +0.036 at degree 5. It is the XYZ-fitted column that shows the	470

overfitting signature, its gap widening from +0.121 at degree 3 to +0.226 at degree 4. Every	471

value reported here is out-of-sample in any case; the train/test gap is quoted only to show	472

that the degree-4 fit is not being propped up by memorization.	473

Table 8. Degree sweep on CMYKOGV-7 (7 inks, 3,302 analyzed samples, grouped 5-fold cross-validation). “Rows/coefficient” is the training-fold size divided by the 3× terms-per-channel co-efficients actually fitted. “Gap” is the held-out median minus the in-fold training median, av-eraged over the five folds, for the cube-root column. The Gaussian process fitted in the same space scores 0.160 on this dataset. The degree-3 and degree-4 medians reproduce the poly3, poly3_cbrt, poly4 and poly4_cbrt entries of journal/results/CMYKOGV-7/summary.csv. Source: journal/results/CMYKOGV-7/degree_sweep.csv.

| Degree | Median, XYZ | Median, cbrt | Terms/channel | Rows/coefficient | Gap (cbrt) |
| --- | --- | --- | --- | --- | --- |
| 2 | 9.953 | 2.656 | 36 | 24.5 | +0.013 |
| 3 | 5.386 | 0.830 | 120 | 7.3 | +0.029 |
| 4 | 2.080 | 0.272 | 330 | 2.7 | +0.029 |
| 5 | 0.506 | 0.126 | 792 | 1.1 | +0.036 |

487

## Newsprint: Within-Run, Cross-Run, and Leave-One-Out	488

Three experiments probe how well a model characterizing one printer generalizes to	489

another, using the 13 nnewsprint press runs described in Section 3.1	490

(1,485 CMYK samples per run). Three training conditions are compared: (A) within-run,	491

5-fold cross-validation on a single press run (the best case, a model fitted to that specific	492

press, and the only condition run for the full 14-model registry); (B) cross-run, trained on one	493

run and tested on another with no shared training data (all 13 × 12 = 156 ordered pairs);  494

and (C) leave-one-run-out, trained on 12 pooled runs and tested on the 13th. Conditions (B)	495

and (C) use a four-model subset of Gaussian process, third-order polynomial, SVM and	496

deep MLP: winner, classical baseline and two mid-field representatives from Table 4.

In condition (C) only, the 2,000-row cap on Gaussian process training folds	498

(Section 3.3)

significantly reduced the amount of data seen by the model.	502

Table 9. Median ∆E00 per model across the three newsprint generalization experiments (13 press runs). Within-run and leave-one-out values are the median of each run’s own median ∆E00 across the 13 runs, and are therefore individual CSV entries; cross-run values are the median across the 156 ordered train/test press pairs, an even count, so each is the midpoint of the two central entries. All models are fitted on XYZ; the fitting-space correction of Section 4.5 has not been applied to this experiment.   Source:   journal/results/ifra/{within_run,cross_run,leave_one_out}.csv.

| Model | Within-run (A) | Cross-run (B) | Leave-one-out (C) |
| --- | --- | --- | --- |
| Gaussian Process | 0.899 | 3.844 | 3.155 |
| SVM | 1.073 | 3.874 | 3.008 |
| Polynomial (3rd order) | 1.416 | 4.167 | 3.301 |
| MLP (Deep) | 1.614 | 4.132 | 3.625 |

Within-run (A), Gaussian process regression is the best of the 14 504 models on 12 of the 13 runs. One failure mode is worth recording: 506 initialized with a near-zero WhiteKernel (10−5, two orders of magnitude below newsprint’s 507 measured noise), the kernel’s marginal-likelihood optimization enters a local optimum  508 in which the fitted RBF length-scale collapses to its lower bound and the model reverts  509 to predicting close to the training-set mean color, putting GP’s within-run median near 510 18.8, an order of magnitude worse than every other model. That is a property of the 511 optimization, not of Gaussian processes: the neutral initialization, widened bounds and 512 restarts of Section 3.2 find the healthy optimum on every dataset, with fitted noise levels 513 matching each dataset’s measured duplicate-pair repeatability.	514

The substantive story is that press-to-press variation dominates model choice: ev-  515

ery model’s cross-run error (B, 3.844–4.167 median regardless of model) is 2.6–4.3× its	516

own within-run error (A), and no amount of model sophistication closes that gap. This	517

corroborates, from the modeling side, what Nussbaum and Hardeberg [39] found from	518

the process-control side on eight coldset newspaper presses, where inter-press variation	519

exceeded the difference between custom and standard ICC profiles.

532

Figure 4. Median-of-medians ∆E00 under the three newsprint generalization conditions (within-run, cross-run, leave-one-run-out) for third-order polynomial regression, SVM, deep MLP, and Gaussian process regression. Within-run values are per-press-run 5-fold cross-validation; cross-run pools the 156 ordered press pairs. Generated from the results CSVs by fig_ifra_generalization.py.

## Scaling to n > 4 Colorants	533

This section answers question (b) of Section 1 (whether machine learning can handle	534

n > 4 colorant systems, where classical polynomial and LUT-based methods do not scale)	535

on the three datasets extending the ladder to n = 5 and n = 7 (Table 1): the five-ink KCMYG	536

set and the two independent seven-ink sets, CMYKOGV and CMYKOGB (Section 3.1).	537

Table 10 reports median, 95th-percentile, and maximum ∆E00 for the full model registry,	538

the two ∆E00-refined polynomial variants of Section 4.4, and the four fitting-space variants	539

of Section 4.5, under the grouped, deterministic cross-validation protocol of Section 3.3.	540

Because Section 4.5 showed both the fitting space and the degree cap of the classical baseline	541

to be costly, the result below is stated twice, against the uncorrected and the corrected	542

baseline; the two differ by an order of magnitude.	543

Table 10. Median, 95th-percentile, and maximum ∆E00 per model for the n = 5 (KCMYG, 2,214 sam-ples) and n = 7 (CMYKOGV, 3,302 analyzed samples; CMYKOGB, 2,000 samples) datasets. Grouped 5-fold cross-validation with every sample predicted exactly once (Section 3.3); ∆E00 on denormalized XYZ (D50, 2°). Models sorted by KCMYG-5 median; the four fitting-space variants of Section 4.5 are marked †. Source: journal/results/{KCMYG-5,CMYKOGV-7,CMYKOGB-7}/summary.csv.

KCMYG-5	CMYKOGV-7	CMYKOGB-7

| Model | Median | P95 | Max |  | Median | P95 | Max |  | Median | P95 | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gaussian Process (cbrt)† | 0.764 | 2.067 | 16.172 |  | 0.160 | 0.677 | 5.713 |  | 0.977 | 2.996 | 14.270 |
| Polynomial (4th order, cbrt)† | 0.853 | 2.118 | 39.488 |  | 0.272 | 1.189 | 18.612 |  | 1.132 | 3.843 | 24.096 |
| Gaussian Process | 0.867 | 2.267 | 31.738 |  | 0.249 | 4.497 | 49.757 |  | 1.280 | 8.066 | 44.305 |
| Polynomial (3rd order, cbrt)† | 1.024 | 2.389 | 29.230 |  | 0.830 | 2.982 | 27.979 |  | 1.611 | 7.091 | 30.274 |
| Polynomial (4th order)† | 1.057 | 3.289 | 55.860 |  | 2.080 | 13.800 | 57.988 |  | 1.698 | 19.662 | 48.974 |
| SVM | 1.185 | 6.275 | 38.741 |  | 2.138 | 11.259 | 27.735 |  | 1.776 | 13.228 | 42.827 |
| Poly3 + ∆E00 (Powell) | 1.405 | 5.293 | 59.035 |  | 3.104 | 12.482 | 48.585 |  | 2.979 | 19.324 | 49.052 |
| Polynomial (3rd order) | 1.457 | 7.928 | 65.742 |  | 5.386 | 25.150 | 59.833 |  | 3.332 | 24.845 | 48.974 |
| Poly3 + ∆E00 (Nelder–Mead) | 1.464 | 7.239 | 65.888 |  | 5.375 | 21.828 | 52.271 |  | 3.324 | 22.758 | 47.049 |
| k-NN | 1.871 | 4.935 | 54.927 |  | 3.669 | 9.213 | 28.874 |  | 2.560 | 8.022 | 42.641 |
| MLP (Deep) | 1.970 | 9.030 | 35.677 |  | 3.579 | 14.126 | 38.548 |  | 2.536 | 15.106 | 39.321 |
| MLP (Shallow) | 2.013 | 11.520 | 34.424 |  | 3.380 | 14.761 | 43.884 |  | 2.985 | 18.080 | 37.836 |
| Random Forest | 2.237 | 6.925 | 37.295 |  | 3.774 | 9.512 | 29.535 |  | 2.322 | 8.197 | 69.308 |
| Gradient Boosting | 2.825 | 9.941 | 40.629 |  | 3.828 | 16.099 | 35.012 |  | 3.916 | 16.899 | 44.962 |
| Decision Tree | 4.780 | 14.196 | 40.780 |  | 5.337 | 15.833 | 38.032 |  | 5.038 | 15.471 | 85.477 |
| Lasso | 10.681 | 26.169 | 50.045 |  | 14.937 | 27.875 | 50.410 |  | 11.529 | 30.845 | 49.111 |
| Elastic Net | 10.709 | 25.476 | 50.057 |  | 14.967 | 27.829 | 48.606 |  | 11.440 | 31.143 | 49.817 |
| Ridge | 10.730 | 24.982 | 50.271 |  | 15.009 | 27.530 | 44.107 |  | 11.244 | 31.188 | 50.696 |
| PCR | 10.730 | 24.982 | 50.271 |  | 15.009 | 27.530 | 44.107 |  | 11.244 | 31.188 | 50.696 |
| PLSR | 13.572 | 31.858 | 52.912 |  | 18.327 | 37.555 | 63.530 |  | 17.615 | 37.903 | 59.849 |

The uncorrected comparison.	544

Figure 5 gives the registry results on the full n = 3 → 4 → 5 → 7 ladder. Third-order	545

polynomial regression fitted on XYZ

degrades from a median of 0.268 at	547

n = 3 (PC10, Table 3) through 0.942 at n = 4 and 1.457 at n = 5 to 5.386 (CMYKOGV-7) and	548

3.332 (CMYKOGB-7) at n = 7: roughly an order of magnitude over the ladder. Gaussian	549

process regression fitted on XYZ stays at or below a median of 1.280 on every rung.

This suggests the classical method collapses at high ink counts while the machine	551

learning method holds, with a factor of 21.6 between them at seven inks on CMYKOGV	552

(5.386 against 0.249).

555

568

Key points	569

1) A Gaussian process is still the best-median model on all three	570

n > 4 datasets and, fitted in cube-root space, on 8 of the 9 datasets in this paper (in XYZ	571

on the ninth, FOGRA51-CMYK, by 0.001 ∆E00).

2) Its advantage in the error tail	572

at high ink count is much larger than its advantage in the median and does not shrink	573

under the correction: on CMYKOGV-7 it holds P95 0.677 and maximum 5.713 against the	574

corrected polynomial’s 1.189 and 18.612, and on CMYKOGB-7 2.996 and 14.270 against	575

3.843 and 24.096. For a characterization model the worst patch often matters more than the	576

typical one, and that is where the gap remains.

3) Most usefully to a practitioner, 	577

the Gaussian process reaches those numbers with no choices to make: the polynomial’s	578

competitiveness at n = 7 depends on selecting a degree and a fitting space, and getting	579

either wrong is expensive.

584

Figure 5. Median ∆E00 versus number of inks n ∈ {3, 4, 5, 7} for Gaussian process regression, third-order polynomial regression, and three mid-field models (log scale), all fitted on XYZ. Each rung is an independent dataset and printing system, so each model’s own trend, not levels across rungs, is the meaningful comparison; the second seven-ink system (CMYKOGB) is drawn as a dashed branch. Linear-family models are omitted (medians 6.5–18.3 across these datasets). This figure shows the uncorrected comparison: the polynomial curve is the degree-3 XYZ fit, and Section 4.5 shows that most of its rise between n = 4 and n = 7 is attributable to the fitting space and the degree cap. At degree 4 in CIELAB the same curve would read 0.085 (n = 3, PC10), 0.112 (n = 4, PC10), 0.853 (n = 5), and 0.272 and 1.132 at n = 7, i.e. essentially flat across the ladder. Generated from the results CSVs by fig_ncolor_ladder.py.

595

One number in Table 10 demands a caveat rather than a headline: Gaussian process  596

regression’s median on CMYKOGV-7 is better than its median on the five-ink KCMYG set, in	597

both fitting spaces (0.249 against 0.867 in XYZ; 0.160 against 0.764 in cube-root space). This	598

is not evidence that seven inks are easier than five. In the CMYKOGV chart, 84% of patches	599

use at most four nonzero inks, so the chart is a union of densely sampled low-dimensional	600

wedges that an RBF-kernel GP interpolates extremely well, and it is also the largest of the	601

three datasets (3,302 analyzed samples).

603

613

## Large Language Models as Equation Writers: An Exploratory Probe	614

Scope.	615

This subsection is an exploratory probe, not a benchmark, and it should not be	616

read as one.  Each row of Table 11 is a single query to a single model (n = 1); the	617

conditions differ from row to row in ways that demonstrably matter more than the	618

model identity, including whether the model was given a code interpreter and whether	619

its reasoning mode was enabled; and every row concerns one three-ink chart, PC10- 620 CMY, so nothing here bears on the n > 4 question that motivates the rest of the paper. 621 Model identifiers, providers, temperature, token usage and cost are recorded per row in  622

journal/results/llm/equation_summary.csv, and the verbatim prompt and every raw	623

response are archived, so the probe is reproducible even though it is not statistically pow-	624

ered. We report it because the effect sizes are large enough to be informative in spite of that,	625

and because one of its findings is methodological rather than colorimetric.	626

The task.	627

For the LLMs we asked not for per-patch	628

predictions but for a portable equation: “Generate an equation that transforms any coordinate	629

in data set A into a coordinate in data set B. The equation should be as simple as possible,	630

and avoid exponents greater than 3. The success criterion is minimisation of average	631

and maximum differences between CIELAB values in data set B and those estimated	632

by the equation, as defined by the CIEDE2000 equation.” Each model was shown 150	633

(c, m, y) → (X, Y, Z) training pairs in its prompt and asked to return three expressions	634

in c, m, y. Answers were parsed by a tokenizer whitelist and sympy (model text is never	635

evaluated as code) and scored on 100 held-out patches with the same ∆E00 path as every	636

classical model in this paper. The comparator, poly3-ls-local, is a least-squares cubic	637

fitted on exactly those 150 rows and scored on exactly those 100 patches, so the LLM and	638

the classical method see identical data.	639

Table 11. Large language models asked to write a CMY→XYZ characterization equation, PC10-CMY, 150 in-prompt training pairs, scored on 100 held-out patches with this paper’s ∆E00 path. “Degree” is the total degree of the expanded expression; the brief asked for exponents no greater than 3. Source: journal/results/llm/equation_summary.csv; the web/code-execution row is scored from the archived answer in journal/llm/raw/equation_harness/ by the same parser and metric.

| Model | Condition | Median | P95 | Max | Terms | Degree |
| --- | --- | --- | --- | --- | --- | --- |
| Claude Fable 5 | web UI, with code execution | 0.082 | 0.241 | 0.752 | 2,542 | 27 |
| Least-squares cubic (ours) | not an LLM; same 150 rows | 0.234 | 0.917 | 3.052 | 57 | 3 |
| GPT-5.6 Sol | API, reasoning enabled | 3.070 | 7.312 | 10.318 | 192 | 9 |
| Claude Opus 5 | API, reasoning disabled | 14.291 | 24.081 | 39.999 | 192 | 9 |
| DeepSeek V4 Pro | API, reasoning disabled | 23.764 | 42.444 | 49.739 | 39 | 3 |
| Claude Haiku 4.5 | API, reasoning enabled | 28.781 | 46.822 | 57.556 | 33 | 3 |
| Claude Fable 5 | API, reasoning enabled | no equation returned (1,600 and 78,000 completion tokens) |  |  |  |  |
| DeepSeek V4 Pro | API, reasoning enabled | no equation returned (2,400 and 8,000 completion tokens) |  |  |  |  |

Unaided, no model is competitive.	640

The best equation produced by a model reasoning from the numbers alone is GPT-5.6	641

Sol’s, at median 3.070 ∆E00, 13× worse than the least-squares cubic fitted on the same	642

150 rows (0.234), with a maximum of 10.318 against 3.052. The brief’s success criterion,	643

minimizing both average and maximum difference, is not met by any unaided model. The	644

remaining three unaided answers are worse by a further order of magnitude, and inspection	645

shows why: DeepSeek V4 Pro’s cubic has near-proportional coefficients across the three	646

channels, so it cannot produce chroma at all and predicts a mid gray for a saturated yellow	647

patch. Asking a language model to write a characterization transform is not a substitute for	648

fitting one, and the gap is not marginal.	649

Given a code interpreter, one model beat the cubic: an automation result, not a	650

color-science one.	651

The top row of Table 11 was produced by Claude Fable 5 in a browser session with	652

code execution available. It returned per-channel expressions of the form (cubic in c, m, y)3	653

carrying coefficients such as 4.40094852422, and it scores median 0.082 ∆E00, roughly three	654

times better than our own cubic. That precision is the signature of a numerical fit, and	655

the archived session confirms it: the model wrote and ran code, fitted the coefficients	656

numerically, and reported the result. What the row measures is therefore an agent with	657

a least-squares library, not a language model’s colorimetric reasoning, and it is reported	658

here only to make the point that the two are indistinguishable from the answer text alone.	659

Attempts to obtain a clean tool-free answer through a command-line agent instead of a	660

browser failed repeatedly: with successive tool categories disabled, the agent found a	661

computation channel each time (187 and 74 tool invocations in the first two attempts)	662

and fitted the coefficients numerically rather than reasoning them out. Any evaluation	663

of language models on quantitative tasks that queries them through an agentic harness,	664

without auditing the transcript for tool calls, is measuring the harness; that is the probe’s	665

methodological finding.	666

Accuracy was bought by breaking the constraint.	667

The brief asked for an equation “as simple as possible” with no exponent above 3,	668

and the ranking in Table 11 is almost inverse to compliance with it. The two answers that	669

respect the letter and the spirit of the constraint (DeepSeek’s 39-term cubic, and Haiku’s	670

33-term expression of total degree 3 with no single variable raised above the square) are	671

the two worst. GPT-5.6 Sol and Claude Opus 5 keep every single-variable exponent at 3	672

but multiply three per-ink cubics together, giving an expanded total degree of 9 and 192	673

terms. The code-execution answer nests a cubic inside a cube: per-variable exponent 9, 674 total degree 27, 2,542 terms once expanded, which is neither simple nor portable in any 675 useful sense. The more accurate the answer, the more thoroughly the simplicity constraint 676 was circumvented.	677

Reasoning budget is the binding constraint, and it is not uniform across providers.	678

Two of the models never returned an equation at all. Claude Fable 5, with reasoning	679

enabled, returned no equation at either completion budget: at 1,600 tokens the response hit	680

the length limit mid-derivation with no visible content (238 archived reasoning tokens),	681

and at 78,000 tokens it spent essentially the entire allowance on reasoning (77,999 reasoning	682

tokens) and again returned empty content; DeepSeek V4 Pro did the same at 2,400 and	683

8,000 before returning an answer only once reasoning was disabled, which is why its	684

row is labeled as such and why its coefficients read as recalled rather than fitted. GPT-	685

5.6 Sol, by contrast, converged and answered within 2,747 completion tokens, of which	686

2,358 were reasoning. On a task with a well-defined stopping point, one frontier model	687

terminated inside 3,000 tokens while another had not terminated inside 78,000. With n = 1	688

per condition this is an observation rather than a measurement, but on this task completion	689

cost was dominated by whether the model terminated, not by whether it was right.	690

Relation to the earlier direct-prediction pilot.	691

An earlier pilot on the same 100 held-out patches asked two OpenAI models to	692

predict XYZ directly rather than to write an equation, with 400 in-context examples	693

and temperature-0 decoding (journal/results/llm/PC10-CMY_summary.csv).  GPT-4o	694

reached a median of 3.034 ∆E00 (P95 10.642, max 14.621) and GPT-4o-mini 9.445 (P95 29.928,	695

max 44.792); all 200 responses parsed. GPT-4o’s per-patch prediction and GPT-5.6 Sol’s	696

portable equation therefore land in the same place, 3.034 against 3.070, and both are an	697

order of magnitude behind least squares on the same rows. That pilot’s holdout predates	698

the de-duplication policy of Section 3.3 and was drawn from a recipe-de-duplicated pool	699

of the as-received 818 rows (its split code drops repeated recipes, a stricter rule than the	700

byte-identical policy adopted later) and has not been re-drawn, so its numbers are quoted	701

for context only and are not comparable in protocol to the cross-validated tables above	702

(Figure 6).	703

Figure 6. The earlier direct-prediction pilot: median ∆E00 for GPT-4o and GPT-4o-mini predicting XYZ per patch (single 100-sample holdout, 400 in-context examples) alongside the classical/ML model set and the two ∆E00-refined polynomial variants (5-fold cross-validation, 795 analyzed samples), PC10-CMY. The two evaluation protocols differ as described in the text; log scale, dashed line at ∆E00 = 1.

## External Validation Against an Independent Tool	704

CharData Viewer [40] (v1.18.0), a public browser-resident characterization-data ex-	705

plorer, ships the same standard datasets this study uses and reports its own polynomial	706

fit statistics, which makes an external cross-check possible on identical data.

Results are shown in Figure 7, where it can be seen our polynomial and cube-root GP methods perform better than CharData and our XYZ GP methods. 	712

728

743

Figure 7. Mean and maximum ∆E00 on identical data: CharData Viewer’s degree-4 polynomial fit versus this work. The protocols differ and are not interchangeable: the external tool’s statistics are resubstitution (the model is fitted on all loaded rows and evaluated on the same rows, which is systematically optimistic), whereas ours are 5-fold cross-validated out-of-sample errors. The comparison is meaningful only as a floor: our numbers are produced under the harder protocol. Source:      journal/results/colourbill/{colourbill_fit_stats,poly_lab_cv,comparison}.csv.

- Discussion	744

The baseline, before either question.

Section 4.5 establishes	746

that the classical polynomial

was fitted on XYZ and capped at third order, and that both	748

restrictions are costly: correcting the space alone improves the polynomial’s maximum	749

error on 9 of 9 datasets and its median on 8 of 9, and correcting both takes its median	750

on the seven-ink CMYKOGV chart from 5.386 to 0.272 ∆E00. Section 4.5 establishes the	751

mechanism and shows the cube root not to be privileged among compressive transforms;	752

none of this is new as technique (Section 2), and its consequence is that both answers below	753

are narrower than an uncorrected comparison would suggest.	754

# Question (a): does machine learning match or beat classical polynomial regression	755

at n ≤ 4? Yes, but by about a factor of two rather than the order of magnitude the	756

uncorrected comparison suggested. Gaussian process regression is the best-median model	757

on all three printer datasets at both n = 3 and n = 4 (Tables 3–4); against the best	758

polynomial available at any degree and fitting space it is 2.0–2.4× better in median, and	759

against the corrected degree-3 CIELAB polynomial 3.3–8.2× better. The 6× and 12–17×	760

margins over the degree-3 XYZ fit reflect the baseline’s fitting space as much as the model’s	761

merit. The result is insensitive to the fold-assignment scheme (Section 4.3), survives an	 762 independent external cross-check (Section 4.9), and was reproduced to within 0.003 ∆E00	763 by a clean-room reimplementation, on the pre-deduplication results set (Section 3.3). On	764 the gray-component-replacement motivation behind the question, the reading is likewise	765 narrower: the Gaussian process handles the added K channel as a fourth input dimension	766 with no special-casing, degrading ×1.2 in median error, but so does the polynomial once it	 767 is fitted in the same space (×1.5 at degree 3, ×1.3 at degree 4), against ×3.5 for the XYZ fit	768 (Figure 2). The claim that a fixed-degree basis cannot absorb K’s interaction terms while	 769 a kernel adapts is not supported once the fitting space is controlled for; what remains		770 is a level difference. A second, narrower result refines rather than replaces the classical	771 polynomial: optimizing its coefficients directly on ∆E00 (Section 4.4) cuts worst-case error	 772 16–51% at n = 3 at essentially unchanged median, and at n = 4 improves the median by	 773 4–9% and the 95th percentile by 19–27%, though a change of fitting space, which is free,	774 does more (0.942 → 0.219 on PC10-CMYK, against Powell’s 0.867). One caveat: the newsprint	775 results (Section 4.6) show press-to-press variation, not model choice, dominating error		776 once training and test data come from different presses. GP is the best within-run model	777 on newsprint too (median-of-medians 0.899, at the press noise floor), but that advantage	778 vanishes under cross-run and leave-one-out evaluation, where it is indistinguishable from	 779 SVM, polynomial regression and a deep MLP.			781 Question (b): can machine learning handle n > 4 colorant systems where classical	782 methods do not scale? Yes; and so can classical polynomial regression, given a perceptual	783 fitting space and a fourth degree. Gaussian process regression remains the best-median	784 model on the five-ink KCMYG set and both seven-ink systems, at 0.764, 0.160 and 0.977	785

∆E00 fitted in cube-root space. But the corrected polynomial follows at 0.853, 0.272 and 786 1.132, factors of 1.1, 1.7 and 1.2 rather than the factor of 21.6 that the degree-3 XYZ baseline 787 implied on CMYKOGV-7; on KCMYG-5 and CMYKOGB-7 it beats the XYZ-fitted Gaussian 788 process (0.853 against 0.867; 1.132 against 1.280). The claim is therefore not 789 that classical methods fail beyond CMYK, but that they need two things in order not to (a 790 fitting space and a degree), and that both must be chosen, whereas the Gaussian process 791 requires neither and additionally keeps a substantial margin in the error tail, where a 792 characterization model is often judged (CMYKOGV-7: P95 0.677 and maximum 5.713 793 against 1.189 and 18.612). The classical route does still fail on a third axis, feasibility of 794 direct metric optimization: Nelder–Mead stalls barely off its least-squares initialization 795 at n = 7, silently returning a model no better than the baseline, while Powell improves 796 worst-case behavior but at 41–62 minutes per dataset versus seconds at n ≤ 4. Two caveats 797 bound everything above: the rungs are independent datasets and printing systems, so only 798 each model’s own trend across n is meaningful, not absolute levels between rungs; and the 799 GP’s low CMYKOGV-7 median partly reflects that chart’s structure (84% of patches use ≤4 800 inks) rather than intrinsic seven-ink ease (Section 4.7).	801

Limitations. Every model comparison here (Sections 4.1–4.5 and 4.7) is trained and  802

evaluated within a single substrate per dataset; the newsprint experiments (Section 4.6) are	803

the paper’s only cross-printer evidence, and they cover one substrate class (newsprint)	804

at one colorant count (n = 4). The fitting-space and degree corrections of Section 4.5	805

were applied to the polynomial and to the Gaussian process, the two models the paper’s	806

argument rests on, but not to the remaining twelve; their positions in Tables 3, 4 and 10	807

should be read as XYZ-space results and may improve under the same treatment, so the	808

mid-field ordering is not a settled ranking. Nor has the degree sweep been repeated on	809

datasets other than CMYKOGV-7, so degree 4 is demonstrated not to overfit at 2.7 rows	810

per coefficient on one 3,302-row chart rather than in general. Gaussian process regression’s	811

O(N3) training cost in the number of samples is a deployment concern as datasets grow  812 (the newsprint leave-one-out condition pools ∼17,800 samples), separate from its accuracy 813 advantage; the corrected polynomial’s 0.1 s fit is the relevant counterweight. The language- 814 model probe of Section 4.8 is n = 1 per condition on one three-ink chart and supports no 815 ranking of models.	819

- Conclusions	820

Fitted against the cube root of	824

XYZ (a similar estimator to fitting in CIELAB), the degree-3 polynomial

gives good results, while raised to degree 4 in that	826

space it reaches a median of 0.272 ∆E00 on a seven-ink chart

. Section 4.5 establishes the mechanism and shows the cube root to be merely the	828

transform that coincides with CIELAB, not the best one on most datasets.	829

Against that baseline, Gaussian process regression remains the best-median	830

model on 8 of the 9 datasets, at n = 3, 4, 5 and 7 alike

; on two of the three n > 4 datasets a degree-4 CIELAB polynomial beats the	833

XYZ-fitted Gaussian process. What the Gaussian process retains, and what a practitioner	834

should weigh, is that it requires no choice of degree or fitting space, and that it holds	835

a clear margin in the error tail at high ink count. Refining the polynomial’s coefficients	836

directly on ∆E00 helps at the margins but is largely subsumed by choosing the fitting space	837

correctly, and at n = 7 it either stalls (Nelder–Mead) or becomes hour-scale (Powell). Cross-	838

printer generalization, tested on 13 newsprint press runs, is dominated by press-to-press	839

variation rather than model choice, and pooling training data across presses recovers only	840

20–32% of the gap. An independent public tool, run on byte-identical data, confirms both	841

our ∆E00 arithmetic and that these cross-validated errors match or beat its in-sample fit;	842

by fitting its own polynomial in CIELAB, it also independently corroborates the space	843

correction above. Finally, an exploratory probe of large language models asked to write a	844

characterization equation finds the best unaided answer 13× worse than a least-squares	845

cubic on the same 150 rows; the one model that beat the cubic did so by writing and running	846

code, which measures an agent rather than a language model. The answer to the paper’s	847

central question is therefore affirmative but narrower than expected: machine learning	848

does characterize n > 4 colorant systems well, and so does classical polynomial regression,	849

provided it is given the fitting space and the degree it needs.	850

Author Contributions: Conceptualization, M.H.Z. and P.J.G.; methodology, M.H.Z.; software, M.H.Z.;	851

validation, M.H.Z. and P.J.G.; formal analysis, M.H.Z.; investigation, M.H.Z.; resources, P.J.G.; data	852

curation, M.H.Z.; writing (original draft preparation), M.H.Z.; writing (review and editing), M.H.Z.	853

and P.J.G.; visualization, M.H.Z.; supervision, P.J.G.; project administration, P.J.G.; funding acquisition,	854

P.J.G. All authors have read and agreed to the published version of the manuscript.	855

Funding: This research received no external funding.	856

Institutional Review Board Statement: Not applicable.	857

Informed Consent Statement: Not applicable.	858

Data Availability Statement: The analysis code, experiment scripts and every results CSV under- 859 lying the tables and figures of this study are publicly available at https://github.com/hamzafer/ 860 Evaluating-Machine-Learning-Methods. The characterization datasets themselves are third-party 861 data and are not redistributed there. Three of them are publicly available from their origina-  862 tors: FOGRA51 from the Fogra Research Institute, https://fogra.org/en/downloads/work-tools/ 863 characterisation-data [33]; and the APTEC PC10 and PC11 charts [34] together with the APTEC 864 CMYKOGV seven-ink chart, from the ICC characterization data registry, https://registry.color.org/ 865 [36]. Restrictions apply to the availability of the remaining datasets. The IFRA 2005 newsprint charts 866 are licensed research data, used here under that license; the KCMYG five-ink chart and the Apex 867 CMYKOGB seven-ink chart were obtained from third parties through the second author. These three 868 are not the authors’ to redistribute and are available from the respective providers on reasonable 869 request and with their permission.	870

Acknowledgments: The authors thank William Li and the providers of the five- and seven-colorant	871

characterization data used in Section 4.7. During the preparation of this manuscript/study, the	872

authors used Anthropic Claude (Claude Code command-line agent; Claude Sonnet 5, Claude Opus	873

4.6, Claude Opus 5 and Claude Fable 5 models) for the purposes of implementing the characteriza-	874

tion pipeline and analysis scripts, generating figure-plotting code, and drafting and copy-editing	875

manuscript prose. The authors have reviewed and edited the output and take full responsibility for	876

the content of this publication. This statement concerns AI used as a tool in preparing the work, and	877

is separate from Section 4.8, in which large language models are themselves the object of study.	878

Conflicts of Interest: The authors declare no conflicts of interest.	879

Abbreviations	880

The following abbreviations are used in this manuscript:	881

882

CMY	Cyan, Magenta, Yellow

CMYK	Cyan, Magenta, Yellow, Key (Black)

CMYKOGV	Cyan, Magenta, Yellow, Key, Orange, Green, Violet CMYKOGB	Cyan, Magenta, Yellow, Key, Orange, Green, Blue CV	Cross-Validation

GP	Gaussian Process

ICC	International Color Consortium

LLM	Large Language Model

LUT	Look-Up Table

MLP	Multilayer Perceptron

NTNU	Norwegian University of Science and Technology

SVM	Support Vector Machine (used here as support vector regression, SVR)

∆E00	CIEDE2000 color-difference formula

883

References	884

- Bala, R.; Klassen, R.V. Efficient Color Transformation Implementation. In Digital Color Imaging Handbook; Sharma, G., Ed.; CRC	885

Press: Boca Raton, FL, 2003; pp. 687–725. https://doi.org/10.1201/9781420041484-11.	886

- Boll, H. Color-to-Colorant Transformation for a Seven Ink Process. In Proceedings of the Device-Independent Color Imaging,	887

Proc. SPIE, 1994, Vol. 2170, pp. 108–118. https://doi.org/10.1117/12.173839.	888

- Deshpande, K.; Green, P.; Pointer, M.R. Gamut Evaluation of an n-Colour Printing Process with the Minimum Number of	889

Measurements. Color Research & Application 2015, 40, 408–415. https://doi.org/10.1002/col.21909.	890

- Kang, H.R.; Anderson, P.G. Neural Network Applications to the Color Scanner and Printer Calibrations. Journal of Electronic	891

Imaging 1992, 1, 125–135. https://doi.org/10.1117/12.57526.	892

- Tominaga, S. A Neural Network Approach to Color Reproduction in Color Printers. In Proceedings of the IS&T/SID Color	893

Imaging Conference, Springfield, VA, 1993; Vol. 1993, pp. 173–177. https://doi.org/10.2352/CIC.1993.1.1.art00043.	894

- Su, Z.; Yang, J.; Li, P.; Zhang, H.; Jing, J. Colour Space Conversion Model from CMYK to CIELab Based on CS-WNN. Coloration	895

Technology 2021, 137, 272–279. https://doi.org/10.1111/cote.12529.	896

- Zhan, H.; Zou, Y.; Zhang, Y.; Gong, W.; Xu, F. Color Space Conversion Model from CMYK to CIELab Based on Stacking Ensemble	897

Learning. Color Research & Application 2025, 50, 314–326. https://doi.org/10.1002/col.22971.	898

- Zafar, M.H.; Green, P. Comparison of AI Based Machine Learning Methods with Basic Polynomial Regression for Cross Media	899

Color Reproduction. In Proceedings of the 16th Congress of the International Colour Association (AIC) 2025: Color for Future,	900

Color Association of Taiwan, Taipei, Taiwan, 19–24 October 2025; pp. 59–64. ISBN 978-0-6484724-7-6.	901

- Bala, R. Device Characterization. In Digital Color Imaging Handbook; Sharma, G., Ed.; CRC Press: Boca Raton, FL, 2003; pp. 269–384.	902

https://doi.org/10.1201/9781420041484-5.	903

- Cheung, V.; Westland, S.; Connah, D.; Ripamonti, C. A Comparative Study of the Characterisation of Colour Cameras by Means	904

of Neural Networks and Polynomial Transforms. Coloration Technology 2004, 120, 19–25. https://doi.org/10.1111/j.1478-4408.20	905

04.tb00201.x.	906

- Molada-Tebar, A.; Riutort-Mayol, G.; Marqués-Mateu, Á.; Lerma, J.L. A Gaussian Process Model for Color Camera Characteriza-	907

tion: Assessment in Outdoor Levantine Rock Art Scenes. Sensors 2019, 19, 4610. https://doi.org/10.3390/s19214610.	908

- Hung, P.C. Colorimetric Calibration in Electronic Imaging Devices Using a Look-Up-Table Model and Interpolations. Journal of	909

Electronic Imaging 1993, 2, 53. https://doi.org/10.1117/12.132391.	910

- Vrhel, M.J.; Trussell, H.J. Color Device Calibration: A Mathematical Formulation. IEEE Transactions on Image Processing 1999,	911

8, 1796–1806. https://doi.org/10.1109/83.806624.	912

- He, R.; Xiao, K.; Pointer, M.; Bressler, Y.; Liu, Q. Investigation on Color Characterization Methods for 3D Printer. In Proceedings	913

of the London Imaging Meeting, 2023, Vol. 4, pp. 71–75. https://doi.org/10.2352/lim.2023.4.1.18.	914

- Babaei, V.; Hersch, R.D. N-Ink Printer Characterization With Barycentric Subdivision. IEEE Transactions on Image Processing 2016,	915

25, 3023–3031. https://doi.org/10.1109/TIP.2016.2560526.	916

- Deshpande, K.; Green, P.; Pointer, M.R. Characterisation of the n-colour printing process using the spot colour overprint model.	917

Optics Express 2014, 22, 31786–31800. https://doi.org/10.1364/OE.22.031786.	918

- Shi, L.; Babaei, V.; Kim, C.; Foshey, M.; Hu, Y.; Sitthi-Amorn, P.; Rusinkiewicz, S.; Matusik, W. Deep Multispectral Painting	919

Reproduction via Multi-Layer, Custom-Ink Printing. ACM Transactions on Graphics 2018, 37, 1–15. https://doi.org/10.1145/3272	920

127.3275057.	921

- Chen, D.; Urban, P. Deep Learning Models for Optically Characterizing 3D Printers.  Optics Express 2021, 29, 615.  https:	922

//doi.org/10.1364/OE.410796.	923

- Vacareanu, R.; Negru, V.A.; Suciu, V.; Surdeanu, M. From Words to Numbers: Your Large Language Model Is Secretly a Capable	924

Regressor When Given In-Context Examples. In Proceedings of the First Conference on Language Modeling (COLM), 2024.	925

https://doi.org/10.48550/arXiv.2404.07544.	926

- Shojaee, P.; Meidani, K.; Gupta, S.; Barati Farimani, A.; Reddy, C.K. LLM-SR: Scientific Equation Discovery via Programming	927

with Large Language Models.  In Proceedings of the International Conference on Learning Representations (ICLR), 2025.	928

https://doi.org/10.48550/arXiv.2404.18400.	929

- Hoerl, A.E.; Kennard, R.W. Ridge Regression: Biased Estimation for Nonorthogonal Problems. Technometrics 1970, 12, 55–67.	930

https://doi.org/10.1080/00401706.1970.10488634.	931

- Tibshirani, R. Regression Shrinkage and Selection via the Lasso. Journal of the Royal Statistical Society: Series B (Methodological)	932

1996, 58, 267–288. https://doi.org/10.1111/j.2517-6161.1996.tb02080.x.	933

- Zou, H.; Hastie, T. Regularization and Variable Selection via the Elastic Net. Journal of the Royal Statistical Society: Series B	934

(Statistical Methodology) 2005, 67, 301–320. https://doi.org/10.1111/j.1467-9868.2005.00503.x.	935

- Massy, W.F. Principal Components Regression in Exploratory Statistical Research. Journal of the American Statistical Association	936

1965, 60, 234–256. https://doi.org/10.1080/01621459.1965.10480787.	937

- Geladi, P.; Kowalski, B.R.  Partial Least-Squares Regression: A Tutorial.	Analytica Chimica Acta 1986, 185, 1–17.	https:	938

//doi.org/10.1016/0003-2670(86)80028-9.	939

- Cortes, C.; Vapnik, V. Support-Vector Networks. Machine Learning 1995, 20, 273–297. https://doi.org/10.1007/BF00994018.	940

- Cover, T.M.; Hart, P.E. Nearest Neighbor Pattern Classification.  IEEE Transactions on Information Theory 1967, 13, 21–27.	941

https://doi.org/10.1109/TIT.1967.1053964.	942

- Breiman, L.; Friedman, J.H.; Olshen, R.A.; Stone, C.J. Classification and Regression Trees; Wadsworth: Monterey, CA, 1984.	943

- Breiman, L. Random Forests. Machine Learning 2001, 45, 5–32. https://doi.org/10.1023/A:1010933404324.	944

- Friedman, J.H. Greedy Function Approximation: A Gradient Boosting Machine.  Annals of Statistics 2001, 29, 1189–1232.	945

https://doi.org/10.1214/aos/1013203451.	946

- Rasmussen, C.E.; Williams, C.K.I. Gaussian Processes for Machine Learning; MIT Press: Cambridge, MA, 2006. https://doi.org/10.7	947

551/mitpress/3206.001.0001.	948

- Sharma, G.; Wu, W.; Dalal, E.N. The CIEDE2000 Color-Difference Formula: Implementation Notes, Supplementary Test Data,	949

and Mathematical Observations. Color Research & Application 2005, 30, 21–30. https://doi.org/10.1002/col.20070.	950

- Fogra Research Institute.  FOGRA51 and FOGRA52 Characterisation Data (Version 1.0) [Data set].  https://fogra.org/en/	951

downloads/work-tools/characterisation-data, 2020. Accessed on 29 August 2026.	952

- Advanced Printing Technology Centre Limited (APTEC). APTEC PC10 Cardboard and APTEC PC11 CCNB Characterization	953

Data, 2023 v1. ICC Characterization Data Registry, 2023. Available online: https://registry.color.org/cmyk-registry/aptec_	954

coated_cardboard and https://registry.color.org/cmyk-registry/aptec_ccnb (accessed on 29 August 2026).	955

- Mansencal, T.; Mauderer, M.; Parsons, M.; Shaw, N.; Wheatley, K.; Cooper, S.; Vandenberg, J.D.; et al. Colour 0.4.6, 2024. Python	956

library. Available online: https://www.colour-science.org/ (accessed on 29 August 2026), https://doi.org/10.5281/zenodo.1391	957

7514.	958

- International Color Consortium. ICC Characterization Data Registry, 2025. Available online: https://registry.color.org/ (accessed	959

on 29 August 2026). APTEC CMYKOGV coated chart: https://registry.color.org/cmyk-registry/aptec_cmykogvcoated.	960

- Finlayson, G.D.; Mackiewicz, M.; Hurlbert, A. Color Correction Using Root-Polynomial Regression. IEEE Transactions on Image	961

Processing 2015, 24, 1460–1470. https://doi.org/10.1109/TIP.2015.2405336.	962

- Hong, G.; Luo, M.R.; Rhodes, P.A. A Study of Digital Camera Colorimetric Characterization Based on Polynomial Modeling.	963

Color Research & Application 2001, 26, 76–84. https://doi.org/10.1002/1520-6378(200102)26:1<76::AID-COL8>3.0.CO;2-3.	964

- Nussbaum, P.; Hardeberg, J.Y. Print Quality Evaluation and Applied Colour Management in Coldset Offset Newspaper Print.	965

Color Research & Application 2012, 37, 82–91. https://doi.org/10.1002/col.20674.	966

- Li, W. CharData: ICC Profile and CGATS/IT8 Characterisation Viewer in the Browser, 2026. Available online: https://chardata.	967

colourbill.com/ (accessed on 29 August 2026).

Green, P. "Accuracy of colour transforms", Proc. SPIE 6058, Color Imaging XI: Processing, Hardcopy, and Applications, 605802	968